Experiment 9: Design of Convolutional Layer in Zynq SoC

# Objectives of this Experiment

  - To design and simulate a 3×3 Convolutional Layer using RTL (Verilog HDL) for image processing and Convolutional Neural Network (CNN) accelerator applications.

  - To configure nine 8-bit signed input pixel registers (p0–p8) for the 3×3 input window, nine 8-bit signed kernel coefficient registers (k0–k8), and a 16-bit signed bias input.

  - To implement parallel multiply–accumulate hardware logic computing 9 simultaneous products, accumulating partial sums, and adding bias to produce a 20-bit signed output (y).

  - To construct a comprehensive Verilog testbench (tb_conv2d_3x3), simulate the design in Vivado Simulator (XSIM) across multiple filter test cases (All-ones, Edge detection, and Mixed signed values), and verify output accuracy against theoretical calculations.

Required tools:

Xilinx Vivado Design Suite, Vivado Simulator (XSIM), Verilog HDL, Windows / Linux Operating System.

# Method

Step 1: Project Creation — Open Xilinx Vivado and create a new RTL project. Enter the project name and select the target FPGA device or board.

Step 2: Module Architecture & Ports Declaration — Create a Verilog module named conv2d_3x3 to implement a 3×3 convolution operation. Declare nine 8-bit signed input pixel ports (p0 to p8) representing the 3×3 image window, nine 8-bit signed kernel coefficient ports (k0 to k8), a 16-bit signed bias input (bias), and a 20-bit signed convolution output (y).

Step 3: MAC Logic Implementation — Implement parallel multiply–accumulate logic: compute 9 product terms (m0 = p0 * k0 through m8 = p8 * k8), accumulate the products (sum = m0 + m1 + ... + m8), and add the bias value (y = sum + bias).

Step 4: Testbench Program Writing — Create a testbench file (tb_conv2d_3x3) instantiating the conv2d_3x3 module. Write test cases for: (1) All-ones kernel (p_i = 10, k_i = 1, bias = 5), (2) Edge detection kernel (p4 = 20, p_others = 5, k4 = 8, k_others = -1, bias = 0), and (3) Mixed signed values (p and k containing negative integers, bias = 2).

Step 5: Simulation & Result Verification — Launch Vivado Simulator (XSIM) and execute behavioral simulation. Observe output waveforms for each test case, record the y values, and compare them against manual theoretical calculations.

# Program

## 1. Verilog RTL Code: 3×3 Convolutional Layer

module conv2d_3x3 (
    input  signed [7:0] p0, p1, p2,
    input  signed [7:0] p3, p4, p5,
    input  signed [7:0] p6, p7, p8,     // 3x3 input window
    input  signed [7:0] k0, k1, k2,
    input  signed [7:0] k3, k4, k5,
    input  signed [7:0] k6, k7, k8,     // 3x3 kernel coefficients
    input  signed [15:0] bias,          // 16-bit bias term
    output signed [19:0] y              // 20-bit convolution output
);
    wire signed [15:0] m0, m1, m2, m3, m4, m5, m6, m7, m8;
    wire signed [19:0] sum;

    // Parallel 8-bit x 8-bit Multipliers
    assign m0 = p0 * k0;
    assign m1 = p1 * k1;
    assign m2 = p2 * k2;
    assign m3 = p3 * k3;
    assign m4 = p4 * k4;
    assign m5 = p5 * k5;
    assign m6 = p6 * k6;
    assign m7 = p7 * k7;
    assign m8 = p8 * k8;

    // Accumulation Tree and Bias Addition
    assign sum = m0 + m1 + m2 + m3 + m4 + m5 + m6 + m7 + m8;
    assign y   = sum + bias;

endmodule

## 2. Verilog Testbench Code

`timescale 1ns / 1ps
module tb_conv2d_3x3;
    // Inputs
    reg signed [7:0] p0, p1, p2, p3, p4, p5, p6, p7, p8;
    reg signed [7:0] k0, k1, k2, k3, k4, k5, k6, k7, k8;
    reg signed [15:0] bias;
    // Output
    wire signed [19:0] y;

    // Instantiate Device Under Test (DUT)
    conv2d_3x3 uut (
        .p0(p0), .p1(p1), .p2(p2),
        .p3(p3), .p4(p4), .p5(p5),
        .p6(p6), .p7(p7), .p8(p8),
        .k0(k0), .k1(k1), .k2(k2),
        .k3(k3), .k4(k4), .k5(k5),
        .k6(k6), .k7(k7), .k8(k8),
        .bias(bias),
        .y(y)
    );

    initial begin        // Test Case 1: All ones kernel
        p0=8'd10; p1=8'd10; p2=8'd10;
        p3=8'd10; p4=8'd10; p5=8'd10;
        p6=8'd10; p7=8'd10; p8=8'd10;
        k0=8'd1;  k1=8'd1;  k2=8'd1;
        k3=8'd1;  k4=8'd1;  k5=8'd1;
        k6=8'd1;  k7=8'd1;  k8=8'd1;
        bias = 16'd5;
        #10;

        // Test Case 2: Edge detection-like kernel
        p0=8'd5;  p1=8'd5;  p2=8'd5;
        p3=8'd5;  p4=8'd20; p5=8'd5;
        p6=8'd5;  p7=8'd5;  p8=8'd5;
        k0=-8'd1; k1=-8'd1; k2=-8'd1;
        k3=-8'd1; k4= 8'd8; k5=-8'd1;
        k6=-8'd1; k7=-8'd1; k8=-8'd1;
        bias = 16'd0;
        #10;

        // Test Case 3: Mixed signed values
        p0=8'd10; p1=-8'd5; p2=8'd3;
        p3=8'd2;  p4=8'd1;  p5=-8'd4;
        p6=8'd6;  p7=8'd7;  p8=8'd8;
        k0=8'd1;  k1=8'd2;  k2=8'd1;
        k3=8'd0;  k4=8'd1;  k5=8'd0;
        k6=8'd1;  k7=8'd2;  k8=8'd1;
        bias = 16'd2;
        #10;
        $stop;
    end
endmodule

1. 2D Convolution Formula:
y = ( ∑ (p_i × k_i) for i = 0 to 8 ) + bias

2. Individual Test Case Arithmetic Verification:
• Test Case 1 (All ones kernel):
  m0 = m1 = ... = m8 = 10 × 1 = 10
  sum = 10 × 9 = 90
  y = 90 + 5 = 95

• Test Case 2 (Edge detection-like kernel):
  m4 = 20 × 8 = 160
  m0 = m1 = m2 = m3 = m5 = m6 = m7 = m8 = 5 × (-1) = -5
  sum = 160 + (8 × -5) = 160 - 40 = 120
  y = 120 + 0 = 120

• Test Case 3 (Mixed signed values):
  m0=10*1=10, m1=-5*2=-10, m2=3*1=3, m3=2*0=0, m4=1*1=1, m5=-4*0=0, m6=6*1=6, m7=7*2=14, m8=8*1=8
  sum = 10 - 10 + 3 + 0 + 1 + 0 + 6 + 14 + 8 = 33
  y = 33 + 2 = 35

3. Bit-Width Precision & Growth Analysis:
• Product Bit Width: 8-bit signed × 8-bit signed = 16-bit signed product (m0 to m8).
• Accumulator Growth: Summation of 9 16-bit signed terms requires ⌈log2(9)⌉ = 4 additional guard bits, yielding a 20-bit accumulator (sum).
• Final Output Bit Width: 20-bit sum + 16-bit bias = 20-bit signed output (y) without arithmetic overflow.

Observation table

| Item | Test Scenario / Parameter | Expected Theoretical Output | Simulated Hardware Output | % Deviation / Status |
| --- | --- | --- | --- | --- |
| 1 | Test Case 1 (All-Ones Kernel) | y = (9 * 10 * 1) + 5 = 95 | y = 95 | 0% Error |
| 2 | Test Case 2 (Edge Detection Kernel) | y = (20 * 8) + (8 * 5 * -1) + 0 = 120 | y = 120 | 0% Error |
| 3 | Test Case 3 (Mixed Signed Inputs) | y = 33 + 2 = 35 | y = 35 | 0% Error |
| 4 | Computational Accuracy | 100% MAC Precision | 100% MAC Precision | Verified |

Figure: Output obtained for convolution process

## Conclusion

The 3×3 Convolutional Layer RTL module was successfully designed, implemented, and simulated using Verilog HDL in Xilinx Vivado. The hardware architecture correctly executed 9 parallel 8-bit multiplications, accumulated the results, and added the 16-bit bias term to produce a 20-bit convolution output. The simulation results across all three test cases (All-ones, Edge detection, and Mixed signed values) matched theoretical calculations with 0% error, confirming the correct operational functionality of the convolutional layer hardware for AI workloads.