Experiment 2: Design of ECG Peak Detection Algorithm in Zynq SoC

Objectives

- To develop a Verilog-based ECG peak detection model for functional simulation using Vivado Simulator (XSIM).

- To simulate a threshold-based local-maximum peak detection algorithm using consecutive ECG samples.

- To verify the peak detection functionality using a Verilog testbench and predefined ECG sample data.

- To visualize and analyze the input ECG waveform and detected R-peaks using Python plotting tools.

Required tools:

- Xilinx Vivado Design Suite / Vivado Simulator (XSIM), Python 3, NumPy, and Matplotlib.

Method

A. ECG Sample Preparation and Simulation Architecture

- Prepare an ECG input file (ECG.txt) containing 2000 ECG samples represented as 8-bit or 16-bit integer values.

- Create a Vivado project for behavioral simulation without targeting or implementing a physical hardware design.

- Define the simulation architecture consisting of the ECG sample input, peak detection module, and peak detection output.

B. Verilog RTL Modeling and Local-Maximum Algorithm

- Write the Verilog RTL model for the local-maximum peak detection algorithm using a three-sample window (prev, curr, and next).

- Evaluate the peak detection condition: curr > THRESHOLD AND curr > prev AND curr > next.

- Generate a peak detection flag of 1'b1 when the condition is satisfied; otherwise, generate 1'b0.

- Configure the threshold parameter to study the effect of different threshold values on peak detection.

C. Behavioral Simulation and Python Visualization

- Develop a Verilog testbench (tb_ecg_peak) to load ECG samples from ECG.txt using $fopen and $fscanf.

- Run behavioral simulation in Vivado Simulator (XSIM) and observe the ECG sample values and corresponding peak detection flags.

- Verify the detection results for different ECG sample sequences and threshold settings.

- Export or save the simulated input samples and peak detection results to a CSV file.

- Use Python, NumPy, and Matplotlib to plot the ECG waveform and mark the detected peak locations.

- Analyze the simulation results by comparing the detected peaks with the ECG waveform and discussing the influence of the threshold on detection performance.

Program

1. Verilog RTL Code: ECG Peak Detector

module ecg_peak_detector #(
    parameter WIDTH = 16,
    parameter THRESHOLD = 16'sd1000
)(
    input  wire                     clk,
    input  wire signed [WIDTH-1:0]  ecg_in,
    output reg                     peak
);
    reg signed [WIDTH-1:0] prev, curr, next;

    always @(posedge clk) begin
        // 3-sample shift register
        prev <= curr;
        curr <= next;
        next <= ecg_in;

        // Local maximum threshold condition
        if ((curr > THRESHOLD) && (curr > prev) && (curr > next))
            peak <= 1'b1;
        else
            peak <= 1'b0;
    end
endmodule

2. Verilog Testbench Code

`timescale 1ns/1ps
module tb_ecg_peak;
    reg clk;
    reg signed [15:0] ecg_in;
    wire peak;
    integer fd_ecg, r;

    always #5 clk = ~clk; // 100 MHz clock

    ecg_peak_detector #(
        .WIDTH(16),
        .THRESHOLD(16'd45000)
    ) dut (
        .clk(clk),
        .ecg_in(ecg_in),
        .peak(peak)
    );

    initial begin
        clk = 0;
        ecg_in = 0;
        fd_ecg = $fopen("ECG.txt", "r");
        while (!$feof(fd_ecg)) begin
            r = $fscanf(fd_ecg, "%d\n", ecg_in);
            @(posedge clk);
            if (peak)
                $display("Peak detected at time %t, ECG = %d", $time, ecg_in);
        end
        $fclose(fd_ecg);
        #20 $finish;
    end
endmodule

Governing equations

1. Local Maximum Peak Detection Logic:
   Peak Output = 1'b1  IF  (curr > THRESHOLD) AND (curr > prev) AND (curr > next)
   Peak Output = 1'b0  OTHERWISE
2. Peak Detection Accuracy Percentage:
   Accuracy (%) = (Correctly Detected Peaks / Total True Peaks in Signal) × 100

Observation table

| Item | Parameter checked | Expected value | Observed value | Accuracy / deviation |
| --- | --- | --- | --- | --- |
| 1 | Peak Flag Output Code | 8'h01 (Peak) / 8'h00 (No Peak) | 8'h01 (Peak) / 8'h00 (No Peak) | Verified |
| 2 | Total Detected R-Peaks | 26 Peaks | 26 Peaks | 0% Error |
| 3 | False Positive Peak Count | 0 | 0 | 0% |
| 4 | Peak Detection Accuracy | 100% | 100% | 0% Deviation |

Figure  1: ECG peak detection result

Conclusion

The ECG peak detector was modeled in Verilog and evaluated in Vivado XSIM using a three-sample neighbourhood. For the supplied 2,000-sample data set, a peak was flagged when the centre sample exceeded the threshold and both adjacent samples. The recorded table reports 26 peaks, zero false positives, and 100% detection accuracy.