Experiment 4: Design of image Intensity Inversion circuit in Zynq SoC

Objectives

- To develop a Verilog-based image intensity inversion module for functional simulation using Xilinx Vivado Simulator (XSIM).

- To process a 256 × 256 grayscale image containing 65,536 pixels using a Verilog testbench.

- To implement 8-bit grayscale intensity inversion using the equation (P_{out}=255-P_{in}).

- To verify the inverted pixel values and analyze the input and output signals using Vivado simulation waveforms.

Required tools

- Software: Xilinx Vivado Design Suite, Vivado Simulator (XSIM), and a text editor for preparing Verilog RTL and testbench files.

Method

A. Grayscale Image Pre-processing and Data Formatting

- Prepare a 256 × 256 grayscale image containing 65,536 pixels, with each pixel represented by an 8-bit unsigned integer ranging from 0 to 255.

- Convert the image into a sequential list of pixel values and save the values in image_in.txt, with one pixel value per line.

- Create a Vivado RTL project for behavioral simulation of the image intensity inversion module. FPGA board targeting and hardware implementation are not required.

B. RTL Modeling and Intensity Inversion Logic

- Develop the intensity_inversion Verilog module with an 8-bit pixel input and an 8-bit pixel output.

- Implement the intensity inversion equation:

pixel_out = 8'd255 - pixel_in

- If synchronous operation is required, update the output on the rising edge of the clock when valid_in is high. Include reset and valid_out signaling if specified in the design.

- Verify the inversion operation using representative pixel values, such as:

  - Input 0 → Output 255

  - Input 100 → Output 155

  - Input 128 → Output 127

  - Input 255 → Output 0

C. Behavioral Simulation and Waveform Verification

- Develop a Verilog testbench (tb_intensity_inversion) to read pixel values from image_in.txt using $fopen and $fscanf.

- Apply the pixel values sequentially to the design and allow sufficient simulation time for the output to be generated.

- Record the corresponding inverted pixel values in image_out.txt using $fdisplay.

- Run behavioral simulation in Vivado Simulator (XSIM) and observe the input pixels, clock, reset, valid signals (if applicable), and output pixels in the waveform window.

- Verify that each output pixel satisfies the equation pixel_out = 255 - pixel_in.

- Confirm that all 65,536 pixels are processed correctly and document the simulation results using representative input-output samples and waveform observations.

Program

- Verilog RTL Module

module intensity_inversion (
    input  wire        clk,
    input  wire        rst,
    input  wire [7:0]  pixel_in,     // 0 to 255
    input  wire        valid_in,
    output reg  [7:0]  pixel_out,
    output reg         valid_out
);
    always @(posedge clk or posedge rst) begin
        if (rst) begin
            pixel_out <= 8'd0;
            valid_out <= 1'b0;
        end
        else begin
            if (valid_in) begin
                pixel_out <= 8'd255 - pixel_in;   // Intensity inversion
                valid_out <= 1'b1;
            end
            else begin
                valid_out <= 1'b0;
            end
        end
    end
endmodule

- Verilog Testbench Code

`timescale 1ns/1ps
module tb_intensity_inversion;
    reg        clk, rst;
    reg  [7:0] pixel_in;
    reg        valid_in;
    wire [7:0] pixel_out;
    wire       valid_out;

    integer infile, outfile;
    integer r;

    intensity_inversion UUT (
        .clk(clk),
        .rst(rst),
        .pixel_in(pixel_in),
        .valid_in(valid_in),
        .pixel_out(pixel_out),
        .valid_out(valid_out)
    );

    // Clock Generation (100 MHz / 10 ns period)
    always #5 clk = ~clk;

    initial begin
        clk = 0;
        rst = 1;
        valid_in = 0;
        pixel_in = 0;
        #20 rst = 0;

        infile  = $fopen("image_in.txt", "r");
        outfile = $fopen("image_out.txt", "w");
        if (infile == 0 || outfile == 0) begin
            $display("File open error");
            $finish;
        end

        while (!$feof(infile)) begin
            r = $fscanf(infile, "%d\n", pixel_in);
            valid_in = 1'b1;
            @(posedge clk);   // input accepted
            @(posedge clk);   // output available
            if (valid_out)
                $fdisplay(outfile, "%d", pixel_out);
        end

        valid_in = 0;
        #20;
        $fclose(infile);
        $fclose(outfile);
        $display("Intensity inversion simulation finished.");
        $finish;
    end
endmodule

Governing equations

1. Image Intensity Inversion Relationship: P_out = 255 - P_in

2. Total Data Volume for 256×256 Image: N = 256 x 256 = 65,536

Observation table

| Item | Parameter checked | Calculated value | Simulated / Measured Output | Finding |
| --- | --- | --- | --- | --- |
| 1 | Processed Frame Size | 65,536 Bytes | 65,536 Bytes | 0% Byte Loss |
| 2 | Sample Inversion (Input: 0) | P_out = 255 | P_out = 255 | 0% Error |
| 3 | Sample Inversion (Input: 100) | P_out = 155 | P_out = 155 | 0% Error |
| 4 | Sample Inversion (Input: 255) | P_out = 0 | P_out = 0 | 0% Error |
| 5 | Image Inversion Accuracy | 100% | 100% | 0% Deviation |

Figure 1: Example of image intensity inversion

Conclusion

The intensity-inversion module was tested in Vivado XSIM on a 256 × 256 grayscale frame. Each pixel was transformed using \(P_{out}=255-P_{in}\), so low input intensities became high output intensities and vice versa. The sample values in the observation table follow this relationship.