Experiment 5: Counter Design using Vivado IP

Objectives

  - To configure an 8-bit Binary Counter IP core in Xilinx Vivado with an increment value of 2.

  - To design and configure a 16-bit Binary Counter IP core in down-counter mode with a decrement value of 1.

  - To construct top-level modules and HDL wrappers connecting system clock, reset, enable, and counter output buses.

  - To perform behavioral simulation in Vivado Simulator (XSIM) to verify counter progression, down-counting sequence, and rollover behavior.

Required tools:

Xilinx Vivado Design Suite, Vivado IP Catalog, Vivado Simulator (XSIM).

Method

A. Design of 8-Bit Up-Counter Circuit (Increment of 2)

1. Open Xilinx Vivado and create a new project targeting the appropriate FPGA device.

2. Open Vivado's IP Catalog and add a Binary Counter IP core (`c_counter_binary`). Set output width to 8 bits and set increment value to 2 (connecting constant 2 to increment input).

3. Create a top-level module connecting clock (`CLK`), reset, and 8-bit output (`Q` / `out1`).

4. Write a Verilog testbench (`tb_clock_divider_by4`) to provide a periodic clock signal, apply reset initially, and monitor output count.

5. Launch Vivado Simulator (XSIM) and observe the output waveform to verify that the counter increments by 2 on each rising clock edge (sequence: 0, 2, 4, 6, ..., 254, 0 rollover).

B. Design of 16-Bit Down-Counter Circuit (Decrement of 1)

1. Open Vivado and create a new RTL project. Enter project name and select target FPGA device.

2. Open IP Catalog, search for Binary Counter, customize output width to 16 bits, select down-counter mode, and enable clock and reset options. Generate the IP core.

3. Create an HDL wrapper for the generated IP and connect clock, reset, enable, and 16-bit output signals.

4. Write a Verilog testbench generating a periodic clock, applying initial reset, asserting enable, and initializing output to a known value such as FFFFH.

5. Run behavioral simulation in Vivado Simulator (XSIM) and observe waveform output to verify that the count decreases by 1 on every clock pulse.

Program

1. Verilog Testbench Code for 8-Bit Up-Counter

`timescale 1ns / 1ps
module tb_clock_divider_by4;
    reg clk;
    reg rst;
    wire [7:0] out1;

    // Instantiate 8-bit Binary Counter IP core (Increment by 2)
    c_counter_binary_0 uut (
        .CLK(clk),
        .Q(out1)
    );

    // Clock generation (100 MHz clock)
    always #5 clk = ~clk;

    initial begin
        clk = 0;
        rst = 1;
        #20 rst = 0;
        #100000;
        $stop;
    end
endmodule

2. Verilog Testbench Code for 16-Bit Down-Counter

`timescale 1ns / 1ps
module tb_16bit_down_counter;
    reg clk;
    reg rst;
    reg enable;
    wire [15:0] count_out;

    // Instantiate 16-bit Down Counter IP core
    c_counter_binary_1 uut_down (
        .CLK(clk),
        .SCLR(rst),
        .CE(enable),
        .Q(count_out)
    );

    always #5 clk = ~clk;

    initial begin
        clk = 0;
        rst = 1;
        enable = 0;
        #20 rst = 0;
        #10 enable = 1;
        #1000;
        $stop;
    end
endmodule

Governing equations

1. Up-Counter Modulo & Rollover Calculation:

Max Count = 2^WIDTH - 1 = 2^8 - 1 = 255
For Increment = 2: Maximum even state = 254. Next state: (254 + 2) mod 256 = 0 (Rollover).

2. Down-Counter Modulo & Underflow Calculation:

Initial State = 2^16 - 1 = 65535 (FFFFH).
For Decrement = 1: Sequence follows FFFFH, FFFEH, FFFDH, ..., 0000H, FFFFH.

Observation table

| Item | Parameter checked | Required behaviour | Observed behaviour | Check |
| --- | --- | --- | --- | --- |
| 1 | 8-Bit Up-Counter Step | Increment by +2 | Increment by +2 | Verified |
| 2 | 16-Bit Down-Counter Step | Decrement by -1 | Decrement by -1 | Verified |
| 3 | Up-Count Sequence & Rollover | 0, 2, 4, ..., 254 -> 0 | 0, 2, 4, ..., 254 -> 0 | Verified |
| 4 | Down-Count Sequence | FFFFH, FFFEH, FFFDH, ... | FFFFH, FFFEH, FFFDH, ... | Verified |
| 5 | Operation Stability | Stable Count Transitions | Validated in XSIM Waveform | Pass |

Figure: Upcounting by 2

Figure: Example of downcounting by 2

Conclusion

Vivado Binary Counter IP was used to configure an 8-bit up-counter with a step of two and a 16-bit down-counter with a step of one in the reverse direction. The traces show the even-number sequence wrapping from 254 to zero and the down-counter progressing from FFFFH toward lower values.