Experiment 1:    Design of Clock Divider Circuit using RTL

Objectives:

To design and verify a clock divider by two circuit using RTL (Verilog HDL) that generates a 50 MHz output clock from a 100 MHz input base clock.

To design and verify a clock divider by four circuit using RTL (Verilog HDL) that generates a 25 MHz output clock from a 100 MHz input base clock.

To implement flip-flop toggle logic for divide-by-2 operation and a 2-bit synchronous counter for divide-by-4 operation with active-high asynchronous reset.

To perform behavioral simulation in Vivado Simulator (XSIM), measure output clock periods, and verify frequency division factors.

Required tools:

Xilinx Vivado Design Suite, Vivado Simulator (XSIM), PC or laptop.

Method:

## (a) Generation of 50 MHz Clock from 100 MHz Clock (Clock Divider by 2)

1. Create a new RTL project in Xilinx Vivado targeting the appropriate FPGA device.

2. Add a Verilog module clock_divider_by2 defining ports: input wire clk, input wire rst, and output reg clk_out.

3. Write the RTL code using toggle logic (clk_out <= ~clk_out) triggered on the rising edge of clk or rst.

4. Create a testbench tb_clock_divider_by2 to generate a 100 MHz periodic clock signal (always #5 clk = ~clk;) and apply initial reset.

5. Run behavioral simulation using Vivado Simulator (XSIM) and observe the 50 MHz output clock waveform.

## (b) Generation of 25 MHz Clock from 100 MHz Clock (Clock Divider by 4)

1. Create a new RTL project or add a new Verilog module clock_divider_by4.

2. Define ports: input wire clk, input wire rst, output reg clk_out, and internal 2-bit register reg [1:0] count.

3. Write the RTL code incrementing count on posedge clk and toggling clk_out when count reaches 2'b01.

4. Create a testbench tb_clock_divider_by4 to generate a 100 MHz clock signal and apply initial reset.

5. Run behavioral simulation using Vivado Simulator (XSIM) and observe the 25 MHz output clock waveform.

Program

- Clock Divider by Two Module

module clock_divider_by2 (
    input  wire clk,
    input  wire rst,
    output reg  clk_out
);
    always @(posedge clk or posedge rst) begin
        if (rst)
            clk_out <= 1'b0;
        else
            clk_out <= ~clk_out;
    end
endmodule

- Testbench Program for Clock Divider by Two

timescale 1ns / 1ps
module tb_clock_divider_by2;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by2 uut (
        .clk(clk),
        .rst(rst),
        .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
        rst = 1;
        #20 rst = 0;
        #100;
        $stop;
    end
endmodule

- Clock Divider by Four Module

module clock_divider_by4 (

input  wire clk,

input  wire rst,

output reg  clk_out

);

reg [1:0] count;

always @(posedge clk or posedge rst) begin

if (rst) begin

count   <= 2'b00;

clk_out <= 1'b0;

end else begin

count <= count + 1'b1;

if (count[0] == 1'b1)

clk_out <= ~clk_out;

end

end

endmodule

- Testbench Program for Clock Divider by Four

`timescale 1ns / 1ps
module tb_clock_divider_by4;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by4 uut (
        .clk(clk),
        .rst(rst),
        .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
        rst = 1;
        #20 rst = 0;
        #1000;
        $stop;
    end
endmodule

Observation table

| Item | Parameter checked | Expected value | Observed value | Deviation (%) |
| --- | --- | --- | --- | --- |
| 1 | Input Clock Period (clk) | 10.0 ns | 10.0 ns | 0% |
| 2 | Divide-by-2 Frequency (clk_out1) | 50.0 MHz | 50.0 MHz | 0% |
| 3 | Divide-by-2 Clock Period | 20.0 ns | 20.0 ns | 0% |
| 4 | Divide-by-4 Frequency (clk_out2) | 25.0 MHz | 25.0 MHz | 0% |
| 5 | Divide-by-4 Clock Period | 40.0 ns | 40.0 ns | 0% |
| 6 | Output Duty Cycle | 50% | 50% | 0% |

Calculations

1. Clock Period Relationship: T = 1 / f

2. Frequency Division Formula: f_out = f_in / N

• Divide-by-2 Circuit: f_out1 = 100 MHz / 2 = 50 MHz  ==>  T_out1 = 20 ns

• Divide-by-4 Circuit: f_out2 = 100 MHz / 4 = 25 MHz  ==>  T_out2 = 40 ns

3. Percentage Frequency Deviation: Deviation (%) = | (f_simulated - f_theoretical) / f_theoretical | * 100

Figure 1: Clock division by 2

Figure 2: Clock division by 4

Conclusion

The two clock-divider variants were described in Verilog RTL and tested with Vivado XSIM. The output waveforms corresponded to 50 MHz for divide-by-2 and 25 MHz for divide-by-4, giving periods of 20 ns and 40 ns. These observations agree with the required division ratios.